窗体顶端 ¦ Home ¦ Products ¦ Corporate ¦ Sales ¦ Design Center ¦ 窗体底端

# 3132 and 3133
Ultrasensitive Bipolar Hall-Effect Switches

| Data Sheet (PDF) Selection Guides: Hall-Effect Sensors, Bipolar Hall-Effect Digital Switches, Automotive ICs 窗体顶端 Check Distributor Stock 窗体底端 窗体顶端 when this product is updated 窗体底端 Technical Publications: Understanding Bipolar Hall-Effect Sensors (PDF) Press Releases: Hall-Effect Switch Circuits for High-Temperature Operation General Information: Complete Guide to Data Sheets (PDF) Product Data Sheet Classifications (PDF) | ## Features - 4.5 V to 24 V Operation - Reverse Battery Protection - Superior Temperature Stability - Superior Supply Voltage Stability - Activate with Multi-Pole Ring Magnets - Solid-State Reliability - Small Size - Constant Output Amplitude - Resistant to Physical Stress ## Description These Hall-effect switches are designed for magnetic actuation using a bipolar magnetic field, i.e., a north-south alternating field. They combine extreme magnetic sensitivity with excellent stability over varying temperature and supply voltage. The high sensitivity permits their use with multi-pole ring magnets over relatively large distances. Each device includes a voltage regulator, quadratic Hall voltage generator, temperature stability circuit, signal amplifier, Schmitt trigger, and open-collector output on a single silicon chip. The on-board regulator permits operation with supply voltages of 4.5 to 24 V. The switch output can sink up to 25 mA. With suitable output pull up, they can be used directly with bipolar or MOS logic circuits. ## Functional Block Diagram ## Complete Part Numbers Part Number; Package Type; Temperature; Comments; Samples. UGN3132LT; 3-lead SOT; -20°C to 85°C; 窗体顶端 窗体底端. UGN3132UA; 3-pin SIP; -20°C to 85°C; 窗体顶端 窗体底端. UGS3132LT; 3-lead SOT; -40°C to 125°C; 窗体顶端 窗体底端. UGS3132UA; 3-pin SIP; -40°C to 125°C; 窗体顶端 窗体底端. UGL3132LT; 3-lead SOT; -40°C to 150°C; Contact Local Sales Representative. UGL3132UA; 3-pin SIP; -40°C to 150°C; Contact Local Sales Representative. UGN3133LT; 3-lead SOT; -20°C to 85°C; 窗体顶端 窗体底端. UGN3133UA; 3-pin SIP; -20°C to 85°C; 窗体顶端 窗体底端. UGS3133LT; 3-lead SOT; -40°C to 125°C; 窗体顶端 窗体底端. UGS3133UA; 3-pin SIP; -40°C to 125°C; 窗体顶端 窗体底端. UGL3133LT; 3-lead SOT; -40°C to 150°C; Contact Local Sales Representative. UGL3133UA; 3-pin SIP; -40°C to 150°C; Contact Local Sales Representative. ## Related Applications Automotive > Body Systems > HVAC (commutation main fan) Automotive > Body Systems > Power Seats (seat position) Automotive > Body Systems > Window Lift (pinch detection) Automotive > Powertrain Systems > Transmission (speed) |
| --- | --- |

Home ¦ Site Map ¦ Products ¦ Corporate ¦ Sales ¦ Design Center ¦ On-Line Store ¦ Legal ¦ Help Allegro MicroSystems, Inc., 115 Northeast Cutoff, Worcester, MA 01606 USA, TEL: 1.508.853.5000, FAX: 1.508.853.7861