HlaserOpto CO.,Ltd.

HLASER

9

Http://www.yc-dz.com

HLD780005N6J      780nm   5mW   60℃     Laser Diodes   AUTO  PACKAGE

### ■Specifications

Wavelength:        780nm

#### Light Output:       5mW   CW

Package Type:      TO-18(Φ5.6mm)

■Absolute Maximum Ratings (Tc=25℃)

| Parameter |  | Symbols | Ratings | Units |
| --- | --- | --- | --- | --- |
| Optical Output |  | Po(CW) | 7 | mW |
| Reverse Voltage | Laser | Vr | 2 | V |
|  | PIN PD | Vr (PIN) | 30 | V |
| Operating Temperature |  | Top | -10 ~ +60 | ℃ |
| Storage Temperature |  | Tstg | -40 ~ +80 | ℃ |

■Electrical and optical Characteristics(Tc=25℃)

| Parameter |  | Symbols | Conditions | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Threshold Current |  | Ith | - | 25 | 30 | 35 | mA |
| Operating Current |  | Iop | Po=5mW | 35 | 40 | 50 | mA |
| Operating Voltage |  | Vop | Po=5mW | 1.7 | 1.9 | 2.0 | Volts |
| Differential efficiency |  | η | - | 0.3 | 0.5 | 0.8 | mW/mA |
| Monitor Current |  | Im | Po=5mW | - | 0.2 | - | mA |
| Beam Divergence (FWHM) | Parallel | θ‖ | Po=5mW | - | 11 | - | deg |
|  | Perpendicular | θ⊥ | Po=5mW | - | 37 | - | deg |
| Parallel Deviation Angle |  | △θ‖ | Po=5mW | -2 | - | 2 | deg |
| Perpendicular Deviation Angle |  | △θ⊥ | Po=5mW | -2 | - | 2 | deg |
| Emission Point Accuracy |  | △X,△Y,△Z | Po=5mW | -80 | - | 80 | um |
| Lasing Wavelength |  | λp | Po=5mW | 775 | 785 | 795 | nm |

Im is sorting by custom’s need

◎θ‖ and θ⊥ are defined as the angle within which the intensity is 50% of the peak value.