BLASER CO.,Ltd.

HLD780100H7J  780nm 100mW  70℃ Laser Diodes

### ■Specifications

Wavelength:        780nm

#### Light Output:       100mW   CW (Push  250mW)

Package Type:      TO-18(Φ5.6mm)

■Absolute Maximum Ratings (Tc=25℃)

| Parameter | Symbols | Ratings | Units |
| --- | --- | --- | --- |
| Optical Output | Po(CW) | 110 | mW |
|  | Po(Push) | 250 | mW |
| Reverse Voltage | Vr | 2 | V |
| Operating Temperature | Top | -10 ~ +70 | ℃ |
| Storage Temperature | Tstg | -40 ~ +80 | ℃ |

Push Width 0.1us , Duty50%

■Electrical and optical Characteristics(Tc=25℃)

| Parameter |  | Symbols | Conditions | Min | Typ | Max | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Threshold Current |  | Ith | - | - | 55 | 70 | mA |
| Operating Current |  | Iop | Po=100mW | - | 150 | 165 | mA |
| Operating Voltage |  | Vop | Po=100mW | - | 2.0 | 2.3 | Volts |
| Slope Efficiency |  | η | - | 0.8 | 1.0 | - | mW/mA |
| Beam Divergence (FWHM) | Parallel | θ‖ | Po=100mW | 8 | 10 | 12 | deg |
|  | Perpendicular | θ⊥ | Po=100mW | 16 | 20 | 22 | deg |
| Parallel Deviation Angle |  | △θ‖ | Po=100mW | -2 | - | 2 | deg |
| Perpendicular Deviation Angle |  | △θ⊥ | Po=100mW | -2 | - | 2 | deg |
| Emission Point Accuracy |  | △X,△Y,△Z | Po=100mW | -80 | - | 80 | um |
| Lasing Wavelength |  | λp | Po=100mW | 775 | 780 | 795 | nm |

Im is sorting by custom’s need

◎θ‖ and θ⊥ are defined as the angle within which the intensity is 50% of the peak value.